Waterproof USB Wire Endscope Users Manual

Feature:

- Waterproof wire endoscope

- High resolution with CMOS sensor

- Low lux luminance LED lights can work better when the light is dark

- XP system can watch video directly without install software, Vista or Win7/Win8 need use our software which in the CD

- Can take photo and save it on your PC

- Can record video, and save the video on your PC

- The flexible camera is waterproof that you can inspect area under water, gaps and holes.

Speccification:

| Function items | Spec Description |
| --- | --- |
| Len O.D. | □5.5mm □7mm □9mm □10mm □14.5mm |
| Sensor Size | □1/9 □1/6 □1/5 |
| Resolution | □640*480 □1280*720 □1600*1200 |
| Dimmable LED | □4pcs □6pcs |
| White Comparision | Automatic |
| Exposed control | Automatic |
| Frame Rate | 30fps |
| View angle | 60° |
| LED swith | Adjustable lightness switch |
| Focus Distance | 3cm-10cm |
| I/O Interface | USB 2.0 |
| Video Format | AVI |
| Photo Format | JPEG |
| USB Cable Length | 2M-30M |
| Color | 24 bits color |
| Temperture | Up to 60℃ |

Take Video and Photo

Step1: Put CD to CD-ROM

Step2: Open "ViewPlayCap" ,Select"Accept", then Click "extract",then in your PC desk your will find

Step3: Contact our USB Wire Cam to your PC and open "ViewPlayCap"

Step4: Open the software and click “File”, choose “Set capture file” to set video save file.

Step5: Click “Capture”, if you want to video, click click again to stop it. If you want to photo , Click the “snapshot” for photo.,Photos is saved in the “Administrator → my pictures” automatically.

Package

1. The wire cam unit  x1

2. Color box  x1

3. Users manual  x1

4. CD  x1,

5. Hook  x1 , Magent  x1,  Mirror  x1  (only for 9mm)