H.265+ 4K Hybrid DVR

Feature：

- Support H.265+ encoding, ultra-low bitrates, compatible with H.264 surroundings;

- Support VGA, HDMI full HD output simultaneously;

- Support ONVIF; access third party; Support multiple cloud technology, network penetration, alarm information pushed to your phone and other functions;

- Support web, CMS, center platform management software MYEYE, provide SDK development;

- Support various mobile monitoring (iPhone, Android);

- Remote Access, With DNS (ARSP),easy to monitor remotely

- Support UTC, control signal& video stream transmit through one single coaxial 

- Support XVI/AHD/TVI/CVI/CVBS/IP six signals;

- Support Coaxial audio function

- Support Human detection,Human Face detection,vehicle detection,face check video,car vehicle check video.

Specifications：        

|  | Model | 8504 |
| --- | --- | --- |
| System | Main processor | Technical grade embedded microprocessor TW NT98332 |
|  | Operating system | Embedded LINUX operating system |
|  | System resource | Multi-channel real-time recording synchronously,multi-channel real-time playback, multi-channel multi-user network operation, USB back up. |
| Interface | Operation interface | 16bit real color GUI interface support mouse operation |
|  | Display screen | 1/4/8/9screen (take analog channel as an example,other is base on the selected channel mode) |
| Video | Video standard | PAL/NTSC |
|  | Image encode | H.265+ |
|  | Monitor quality | VGA max 1080P; HDMI at 4K |
|  | Preview Capability | 4*8M@16fps / 4*5M@20fps / 4*4M(6 in 1) |
|  | Encode capability | 4*8M@9fps / 4*5M@16fps / 4*4M@21fps / 4*3M@25fps /4*1080P |
|  | Decode capability | 4*8M@9fps / 4*5M@16fps / 4*4M@21fps / 4*3M@25fps /4*1080P |
|  | Multi-mode video input | Analog(6in1)： 4*8M@9fps；4*5M@16fps； HVR：4*4K（analog）+4*4K（network） NVR：16*1080P;8*5M |
|  | Motion detection | Each channel can set 192（16*12）detection area;can set multi-level of flexibility (for local analog channel only) |
| Audio | Audio compression | G711A |
|  | Bidirectional talk | Support |
| Record and playback | Record mode | Manual>alarm>motion detection>timing |
|  | Local playback | 4ch(analog input mode) |
|  | Video search | Time、date、event、channel search |
| Storage and back up | Video storage | Local HDD 、Network |
|  | Back up mode | Network backup、USB portable HDD、USB imprint、SATA imprint |
| External Interface | Video input | 4CH BNC(1.0Vp-p,75Ω) |
|  | Video output | 1CH VGA，1CH HDMI |
|  | Audio input/output | 1CH input / 1CH output RCA(200-3000mV,5KΩ),can extend to 4CH audio input |
|  | Alarm input/output | N/A |
|  | Network port | RJ45 10M/100M self-adaptive Ethernet port |
|  | PTZ control | 1 RS485 interface,support UTC,coaxial control; |
|  | USB port | 3*USB2.0 |
|  | HDD port | 1 SATA ports（each max to 14TB） |
|  | Wireless port | 3G、WIFI(extension via USB port) |
| Other | Power | 12V/2.5A |
|  | Consumption | <15W（not including HDD) |
|  | Temperature | 0℃-＋55℃ |
|  | Humidity | 10%－90% |
|  | Atmosphere pressure | 86kpa－106kpa |
|  | Dimension | L255*W230*H45MM |
|  | Weight | 1.2KG |