4K 8MP IR Dome PoE IP Camera H.265 Onvif

Model NO.:MEGA 83 EV

Key Features:

Face detection,Humanoid detection, Vehicle detection,and other smart functions;

4K 8MP Black light illumination resolution 3840*2160@16fps, clear and fine images;

2.8-12mm motorized autofocus vari-focal lens.

Adopted 14mil SMD IR led lamp, up to 25-30m IR distance.

Built-in 1ch microphone, built-in PoE/12V 2A power

Support 2D/3D noise reduction, digital wide dynamic;

Advanced H.265AI/H.265+(Compatible with H.265/H.264) video compression, Super low rate, high definition quality of image;

Onvif protocol, support the third party NVR, Support various mobile monitoring(iPhone, Android);

Provide web, CMS, center platform management software XMeye Pro,provide SDK development

Support cloud technology, easy to achieve network penetration, for in-line and alarm information pushed.

Professional anti-lightning, conform to GB/T17626.5 and IEC61000-4-5.

Technical Parameter:

| Model Number | Mega 83 EV |
| --- | --- |
| Camera |  |
| Image CMOS Sensor | 8MP 1/2.8" SONY IMX415 Black light illumination Progressive CMOS sensor |
| DSP | Embedded RTOS,dual-core 32bit DSP(NT98566),pure hard compression ,watch dog |
| Resolution | 8MP Pixel |
| Effective Pixels | 3840*2160@16fps |
| Electronic Shutter | 1/50-1/10000Sec |
| Motion Detection | Support |
| Low illumination | color 0.0001Lux@F1.2, black/white 0.0001Lux@F1.2CMOS |
| Sync System | External |
| Privacy Masking | Support, Up to 4 areas |
| S/N Ratio | ≥52dB |
| White Balance | Auto/Manual |
| Gain Control | Auto/Manual |
| Digital Noise Reduction | Support 2D/3D |
| Wide Dynamic Range | Digital WDR |
| Recording Mode | NVR/CMS/Web |
| Video and Audio |  |
| Video Compression | H.265AI(Compatible with H.265/H.264), support dual stream, AVI;0.1M~8Mpbs variable;1~30f/s variable |
| Video Output | Main stream: 8MP 3840*2160@16fps; 5MP 2880*1616@25fps; 4MP 2560*1440@25fps; 2MP 1920*1080@25fps; sub-stream: 800*448@25fps |
| Audio Processing | G.711,support bidrectional talk,audio&video synchronization |
| Audio input/output | Built-in 1ch High Sensitive Microphone |
| Lens |  |
| Focal Length | 2.8-12mm Motorized Autofocus Varifocal Lens |
| Auto Iris Support | N/A |
| Night Vision |  |
| Infrared LED | 24pcs SMD IR LED, 25-30M IR range |
| Day & Night | Support IR cut |
| Network |  |
| Ethernet | RJ-45 (10/100Base-T)1*RJ45 10/100M adaptive Ethernet port |
| Network Protocol | support RTSP/FTP/PPPOE/DHCP/DDNS/NTP/UPnP etc |
| Smart Phone | Support iPhone, iPad, Android and Windows |
| Browser | Support IE, Chrome, Firefox, Safari |
| Software | Support App(iOS, Android), Window CMS software |
| Intelligence analysis | Face detection, Humanoid detection, Vehicle detection,and supports customization of other smart functions |
| General |  |
| Power | DC12V/2A input, ; Built-in POE 48V1A power supply |
| Power consumption | Led lamp on:≤10W |
| Housing | Al-Alloy Case, White Color Housing |
| Working Temperature | -20℃～65℃ |
| Weatherproof Std. | Support Waterproof, IP66 |
| Dimension | Size: 119(W) x95(H) mm |
| Weight | 0.72 Kg |